Forrest Coleman’s Surf Directions

First one should make sure that the parameters for the alignment are
working well on your data by calling up the GUI dialog in "Section
Overview Processing>Section Overview Robust Rigid Parameters".

---
SURF feature detection

The first step of the algorithm is to extract features from each image
in the stack.  Those parameters which control this process are located
in the SURF parameters box.

PreProcessing Options:
MexicanHat checkbox: Decides whether to run a spatial filter on the
data before extracting features.  The mexican hat is formed by the
summation of two exponential functions with an inner and outer length
scales, with amplitudes scaled such that the total integral of their
sum is zero.  The inner/outer parameters control the length scale of
these exponentials in pixels.  They should be adjusted so that the
blood vessels in the sections are highlighted most strongly.
Generally, the inner parameter should be set to ~1/2 the width of the
blood vessels.

Renormalize: This renormalizes the images such that the mean pixel
value is 128 on a 8-bit scale, and 0/255 are located at +/- X standard
deviations of the mean, where X is the parameter given in the Sigma
box.  This helps bring the level of contrast in thinner sections up to
the same level as it exists in other sections, making the right
Threshold parameter (see below) more consistent from section to
section.

SURF parameters: (for a more detailed explanation of SURF see
http://www.chrisevansdev.com/computer-vision-opensurf.html)
Briefly and vaguely... SURF attempts to find the most prominent
features in images by finding locations with high spatial derivatives,
and writing down a scale invariant descriptor of that feature.  It
does this by taking derivates at different spatial scales.

Initial Downsampling: How much to initially downsample the image
before starting the algorithm.  The higher this value is the faster
the algorithm will run, as the effective image will be smaller, at the
expense of downsampling away features that might exist on finer length
scales.

Octaves: This is how many different spatial scales to test.  Each
Octave represent approximately a factor of 2.

Threshold: This is the threshold which is applied to the derivative at
each spatial scale, it is hard to get a handle on what this means
quantiatively, but qualitatively the higher the threshold, it will
only find the most prominent derivatives.. and the lower the
threshold, it will find more and more features.

Center Fraction: Rather than analyzing the entire image, it will cut
out the middle fraction (how much is set by this parameter).  This
should be chosen to be as small as possible while still having the
same part of the tissue show up somewhere within the cutout from every
section.

When you press the "Test SURF" button, it will run SURF detection on
the two sections specified by the slider bars. It will then show you a
graphical depiction of where the features that were detected are
located on each cutout.  The parameters should be adjusted till it
seems to be detecting 125-500 prominent features in each image.  These
should be mostly blood vessels, and edges of the section.  One should
sample different sections across the stack in order to be confident
that the same threshold produces >125 features in every section.
 ----
RANSAC
The second part of the algorithm involves making comparisons of the
SURF features between pairs of sections, and attempting to find
corresponding features between the sections.

# of top matches: To do this first it takes each of the SURF features
found in the section specified in the top slider (left hand image),
and find the feature in the bottom slider section (right hand image),
which has the closest "descriptor".  It then choses the top N matches
for RANSAC (see below), where N is the parameter specified by this
box.  The larger this number is the larger the RANSAC calculation
takes, and typically correct matches are overrepresented in  the
closest matched features.  I think 150 is a good number, but you are
welcome to make it smaller to make it run faster, or larger to attempt
to find more inliers (see below).

Distance Threshold:  To find which of these matched features are
actually real (many of them are not correct), it runs an algorithm
called RANSAC (see http://en.wikipedia.org/wiki/RANSAC).  Briefly, the
algorithm works by choosing a randomly choosing a small number of
matched features (in this case 2), and then fitting a transformation
that would best bring those features into alignment.  In this case a
rigid transformation (rotation+translation, no scaling change).  It
then applies that transformation to all the other matched features in
the top slider section, and sees how close it brings them to their
matched feature in the bottom slider section. If the distance is
smaller than X% of the width of the cutout image, it will consider
that match to be a "inlier" for that transformation.  The algorithm
simply repeats this process many times.  The iteration which produces
the largest number of inliers is then chosen, and a fit is calculated
using all the "inlier" matches.  Generally, this should be <5%, though
should be made smaller the larger then center fraction is.  The best
way of setting this parameter is to click "test ransac" and see that
when it finds corresponding features the precise localizations of the
features are indeed matched up.  Smaller values of this will reduce
the number of inliers in the best transformation, but produce more
accurate fits.

Max Fractional Scale Factor Change:  This parameter is not relevant
for Rigid transformations, as the scale factor is always 1, but when
other transformation types are implemented, it will discard linear
transformations found during RANSAC which produce determinants that
are signficantly different from 1.  The scale factor change of a
linear transformation can be related to the determinant of the
transformation matrix (delta scale factor (DSF) = sqrt(determinant) ).
    If  abs(DSF-1)>X, where X is the value given by this parameter,
then the transformation will be ignored no matter how many inliers it
has.  One common mode of failure of the algorithm is when many
features are all matched to one feature, or many features from a
single area of the bottom section image.  In this case, one can
produce a transformation which wildly distorts the image, but has many
inliers.  This check avoids finding such false transformations.  This
should be set to reflect the largest actual distortion that is
observed in the dataset between any two pairs of sections... typically
i think 25% change is scale factor is correct.. though perhaps lower.

Model: Right now i have only implemented Rigid transformations, but it
is trivial to include other linear transforms when the rest of
WaferMapper supports them.

When you press the Test Ransac algorithm, it will first extract the
SURF features using the current parameters, then run RANSAC, and
visually show you the results as each "inlier" match will be connected
via lines that extend from one picture to the other. The number of
found inliers will be displayed, as well as the DSF.  Ignore the
estimated total time for now... this is not currently accurate.
RANSAC should be tuned to find >10-15 matched features between any
pair of sections you are expecting to find transformations (see Meta
Alignment below).

---
Meta Alignment
Up to this point we have only described how to find a transformation
between any pair of images.  The next step is to assemble pairwise
connections in order to find a transformation that aligns all the
images.  I have plans to implement 3 different approaches to this
problem, but for now there is only 1 that is implemented "Shortest
Bridge" (see below).

Optimization method: Will choose which method to use.. for now only
select Shortest bridge.

Minimum Inliers for Valid Link: Each algorithm relies upon a
threshhold on the number of inliers found by RANSAC between any pair
of images.  If the found transformation has more than this number of
inliers it will be considered to have found a valid transformation
between those images.  In practice I think this should be atleast 6,
though its safer to make it something higher like 10-15.  By testing
RANSAC, you can get a sense for how many matches RANSAC is finding,
and how many inliers tend to correspond to a correct match by visual
inspection.

Max Delta Sections to Check:  Shortest Bridge worked by starting from
the middle section of the stack (note if the middle section of the
stack is not a valid section, this algorithm will fail.. should
probably make the reference section something you can choose).  It
then attempts to find a transformation between all the sections which
are less than X sections away and this section (where X is specified
by this parameter). .  For those sections where it finds a valid link,
it notes the transformation, connects them in a linked list to the
reference section, and notes them as being a member of "level 1".
Next, the algorithm repeats itself, by finding all the sections that
are within X sections of a member of level 1.  For each of those
sections, it attempts to find a link to a level 1 section, starting
with the nearest section, and proceeding to the farthest section which
is a member of level one and within X sections.  Once it finds a valid
link, it notes the transformation, adds the corresponding pointer to
the linked list, and notes that section as a member of level 2.  This
procedure then iterates until it either finds connections to all the
sections, or no more sections are added, at which point it terminates.

What is left is a linked list and corresponding transformations which
connect all the sections eventually back to the reference section,
using the shortest possible set of "bridges" (or links..perhaps i
should call it shortest link) which are no longer than X sections
long. X should thus be set to be the largest number of delta sections
for which links can be reliably found.  If X is larger than it should
be the algorithm will be inefficient, if it is smaller than many more
bridges will be necessary, and errors can build up (as they do with
the more naive 1>2>3>4>5>6>7... approach).

The transformation for any particular section can thus be constructed
by composing the set of transformations that follow the linked list
back to the reference section.  The reference section thus gets the
identity transformation.

If there are sections which do not have links, they are "orphan"
sections.  When the program runs, these orphan sections will be
displayed to the user, and a series of transformations will be
presented that were rejected for having too few inliers.  The user can
choose to accept a Link which was previously automatically rejected,
or view a different rejected link, or mark that section as "not a
section".  If no links are acceptable, the user will be warned that
this section was not aligned and needs to be manually fixed.

There is no way to test this algorithm without running it completely,
so unlike the other sets of parameters, there is not test button, and
the parameters should be adjusted based upon what the user is seeing
from testing RANSAC on various pairs of sections separated by
different amounts.

---
Running the algorithm
When you are happy with the parameters, close the dialog and their
values will be saved, and will be used when you run the Robust Rigid
Alignment.

...